Advantech AE Technical Sharing Document

| Date | 2021/ 6 / 1 | Release Note | □ Internal ■ External |
| --- | --- | --- | --- |
| Category | ■ FAQ □ SOP | Related OS | Windows、Linux |
| Abstract | Some PCIe cards cannot be detected by CN14/CN31 |  |  |
| Keyword | Network card/WIFI/Bluetooth/BIOS/mPCIe/USB |  |  |
| Related Product | UNO-1483 |  |  |

- Problem Description:

- CN14 and CN31 cannot detect wireless network cards, only CN6 can be used.

- This FAQ can be applied to WIFI (mPCIe) and Bluetooth (USB) network cards

- We take EWM-W188M2(WIFI+Bluetooth) as example

- Brief Analysis and Solution :

- CN14 : It is set to mSATA by default, and mPCIe cards cannot be used.

*For special needs, please contact ACL PAE team.

- CN31 : It needs been adjusted BIOS settings :

  - ChipsetPCH-IO ConfigurationPCI Express ConfigurationPCI Express Root Port X-ASPM[Disable]

  - ChipsetPCH-IO ConfigurationPCI Express ConfigurationPCI Express Root Port X-Detect Non-Compliance Device[Enable]

*Note: CN31 does not provide USB signal and does not support Bluetooth function.

- PCI Express Root Port X default sequence :

  - Port 1 : CN31

  - Port 3 : CN14

*Please select the PCI Express Root Port X of the above options in this order.

- Contact Window:

If you need further information from headquarter, please contact ACL PAE team.